# internship-assesment-

**import cv2

import math

import time

from ultralytics import YOLO

# -------------------------------

# CONFIGURATION

# -------------------------------

MODEL_PATH = "best.pt"  # trained model

IMAGE_WIDTH = 640

FOV_DEG = 70  # approximate camera field of view

# Real-world object heights (meters)

REAL_HEIGHTS = {

"cone": 0.5,

"barrier": 1.0,

"stop_sign": 0.75

}

# -------------------------------

# CAMERA GEOMETRY

# -------------------------------

def compute_focal_length(image_width, fov_deg):

return image_width / (2 * math.tan(math.radians(fov_deg / 2)))

FOCAL_LENGTH = compute_focal_length(IMAGE_WIDTH, FOV_DEG)

def estimate_distance(box_height_pixels, real_height):

if box_height_pixels <= 0:

return None

return (FOCAL_LENGTH * real_height) / box_height_pixels

# -------------------------------

# LOAD MODEL

# -------------------------------

model = YOLO(MODEL_PATH)

# -------------------------------

# VIDEO INPUT (0 = webcam)

# -------------------------------

cap = cv2.VideoCapture(0)

if not cap.isOpened():

print("Error: Cannot access camera")

exit()

# -------------------------------

# MAIN LOOP

# -------------------------------

while True:

start_time = time.time()

ret, frame = cap.read()

if not ret:

break

frame = cv2.resize(frame, (IMAGE_WIDTH, 480))

# Run inference

results = model(frame)[0]

# Process detections

for box in results.boxes:

cls_id = int(box.cls[0])

label = model.names[cls_id]

if label not in REAL_HEIGHTS:

continue

x1, y1, x2, y2 = map(int, box.xyxy[0])

box_height = y2 - y1

distance = estimate_distance(

box_height,

REAL_HEIGHTS[label]

)

# Draw bounding box

color = (0, 255, 0)

cv2.rectangle(frame, (x1, y1), (x2, y2), color, 2)

# Prepare label text

if distance:

text = f"{label}, {distance:.2f}m"

else:

text = f"{label}"

# Draw label

cv2.putText(

frame,

text,

(x1, y1 - 10),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(0, 255, 255),

2

)

# FPS calculation

fps = 1 / (time.time() - start_time)

cv2.putText(

frame,

f"FPS: {fps:.2f}",

(20, 30),

cv2.FONT_HERSHEY_SIMPLEX,

0.7,

(0, 0, 255),

2

)

# Display

cv2.imshow("Detection + Distance Estimation", frame)

if cv2.waitKey(1) & 0xFF == 27:  # ESC to exit

break

cap.release()

cv2.destroyAllWindows()**

train.py (Training Script)

from ultralytics import YOLO

model = YOLO("yolov8n.pt")

model.train(

data="dataset.yaml",

epochs=50,

imgsz=640,

batch=16,

name="detector"

)

dataset.yaml

path: ./data

train: images/train

val: images/val

names:

0: cone

1: barrier

2: stop_sign